DVS System Deployment Process

- Preparation

  - Turn off all unnecessary software and firewall on your PC.

  - Network Switch (layer 3) setup:  (For example: S5048PV3 from H3C with PoE)

    - Turn on: multicast

    - IGMP Snooping on VLAN

      - Dis-select: Enable dropping unknown multicast data

      - Select: Act as lGMP querier, set to 5 seconds.

    - Turn on: DHCP (if necessary)

- Wiring Diagram

- Installation

  - Network Switch with PoE

    - Doesn’t need the 12V/1A power supply adapter.

    - Connects all RJ45 and HDMI cables.

    - Turn on the whole system, and wait for the Network Switch power up.

    - Run PC tool: http://www.lytch.net/MVWP/DVStool/DVSTool-pc.zip

    - Set IP address:

    - Set SS4K30/HS4K30 to work as Encoder OR Decoder

    - Set Source(Encoder)

    - Set Output(Decoder)

    - If you want use iOS/Android, set a WiFi access point as wiring diagram, then install and run iOS/Android tool

- iOS App Store: DVSTool

- Android: http://www.lytch.net/MVWP/DVStool/DVSTool-android.zip

  - Network Switch without PoE

    - Plug in 12V/1A power supply adapter of all nodes.

    - Same as 3.1

- User Interface (PC, iOS, Android)

- LCD Video Wall