Seamless Switching Media Hub

Warning

- Do not expose this device to Rain, Moisture, and

- Dripping

- Only use accessories specified by the manufacture

- Unplug this device during Lightning Storms

- The manual is for reference only, maybe updated

- without further notice

Content

1. Features	3

2. Panel Layout	4

3. EDID and HDCP handle	6

4. Video and Audio	7

5. Specification	8

6. Package Contents	9

7. Mounting dimensions	9

8. RS232 command	9

Introduction

The seamless switching media hub has 2x HDMI2.0 Input ports，1x USB-C(video and audio only) Input port，1x HDMI output port  and  1x HDBaseT  mirror output  port.

It integrated with AC port, LAN port,5V/2A charging port. User can easily manage it via the front buttons, RS232 commands.

- Features

- 2x HDMI and 1x USB-C (video and audio only）Inputs, support input video resolution up to 3840x2160@60

- 1x HDMI and 1x HDBaseT mirror outputs，support output video resolution up to 3840x2160@60

- 1xAC port，2x LAN ports，1x 5V/2A Charging USB-A port

- Support seamless switching

- Support Auto Switching

- Support LPCM, AC3, DD+, DTS, DTS-HD, up to 7.1 audio channel

- Extensive EDID and HDCP control

- Panel Layout

Front Panel

| NO. | Name | Description |
| --- | --- | --- |
| 1 | AC Port | AC Port |
| 2 | 5V/2A | USB-A Port，Support 5V/2A Charging |
| 3 | HDMI 1 | HDMI 1 Input signal selection button（with Led inside） Press and hold this button twice in a row for at least 3 seconds each time will change the output resolution to 3840*2160@60 |
| 4 | HDMI 2 | HDMI2 Input signal selection button（with Led inside） Press and hold this button twice in a row for at least 3 seconds each time will change the output resolution to 3840*2160@50 Hz |
| 5 | USB-C | USB-C Input signal selection button（with Led inside） Press and hold this button twice in a row for at least 3 seconds each time will change the output resolution to 3840*2160@30 Hz |
| 6 | AUTO | Press AUTO button to enable or disable auto switching function (with Led inside） Press and hold this button twice in a row for at least 3 seconds each time will change the output resolution to 1080P@60 Hz |
| 7 | LAN | LAN port，directly connect to the LAN port on the Rear Panel |
| 8 | HDMI 1 | HDMI 1 Input port |
| 9 | HDMI 2 | HDMI 2 Input port |
| 10 | USB-C | USB-C ( video and audio only）input port |
| 11 | BLANK | Press BLANK button to mute the video and audio output. The inside Led will be lit when BLANK function is activated. Press and hold this button twice in a row for at least 3 seconds each time will change the output resolution to 1080P@50Hz |

Rear Panel

| NO. | Name | Description |
| --- | --- | --- |
| 1 | RS232 control | Baud rate 9600, 8 data bits, 1 stop bit, no parity PIN 2 TX ( Hub → PC) PIN 3 RX ( Hub ←PC ) |
| 2 | EDID Switcher | Select EDID mode See the definition of EDID SWITCH for details |
| 3 | HDBaseT out | HDBaseT output port |
| 4 | HDMI out | HDMI output port |
| 5 | RESERVE |  |
| 6 | LAN | LAN port，directly connect to the LAN port on the Front Panel |
| 7 | 100-240VAC | AC Port，Input Voltage Range is 100-240V |
| 8 | L/R | 3.5mm LR audio output port |
| 9 | SERVICE | USB-A port，Used for firmware update |

- EDID and HDCP handle

User can select following EDID modes by RS232 command

| Number | EDID mode | Number | EDID mode |
| --- | --- | --- | --- |
| 1 | AUTO | 10 | 1080P-7.1CH |
| 2 | 4K60-2.0CH | 11 | 720P |
| 3 | 4K60-5.1CH | 12 | 1920x1200 |
| 4 | 4K60-7.1CH | 13 | 1680x1050 |
| 5 | 4K30-2.0CH | 14 | 1600x1200 |
| 6 | 4K30-5.1CH | 15 | 1440x900 |
| 7 | 4K30-7.1CH | 16 | 1360x768 |
| 8 | 1080P-2.0CH | 17 | 1280x1024 |
| 9 | 1080P-5.1CH | 18 | 1024x768 |

Before using RS232 command to select the EDID mode, the FOLLOW SWITCH setting must be OFF (refer the RS232 command list)

When the FOLLOW SWITCH setting is OFF, The default EDID mode is 4K60-2.0

The EDID SWITCH is defined as follows

| Number | EDID mode | Number | EDID mode |
| --- | --- | --- | --- |
| 0,0,0,0 | AUTO | 1,0,0,0 | 1080P-5.1CH |
| 0,0,0,1 | 4K60-2.0CH | 1,0,0,1 | 1080P-7.1CH |
| 0,0,1,0 | 4K60-5.1CH | 1,0,1,0 | 720P |
| 0,0,1,1 | 4K60-7.1CH | 1,0,1,1 | 1920x1200 |
| 0,1,0,0 | 4K30-2.0CH | 1,1,0,0 | 1600x1200 |
| 0,1,0,1 | 4K30-5.1CH | 1,1,0,1 | 1360x768 |
| 0,1,1,0 | 4K30-7.1CH | 1,1,1,0 | 1280x1024 |
| 0,1,1,1 | 1080P-2.0CH | 1,1,1,1 | 1024x768 |

To using the bottom EDID switch to select EDID mode, three steps should be taken:

- The FOLLOW SWITCH RS232 setting must be ON

- Select the desired EDID mode by the bottom EDID switch

- The hub must power on again

The HDMI/HDBaseT output support 3 HDCP options: FORCE-1.4, FORCE-2.2, FORCE-OFF, User can select it by RS232 command

- Video and Audio

The switcher support multiple resolution video input up to 3840x2160@60, and support multiple audio format such as LPCM, AC3, DD+, DTS, DTS-HD, up to 7.1 channel pass through function via HDMI cable

Please note, USB-C input port can only accept LPCM 2.0 audio

The Hub support following video output resolution by a powerful scaling engine

| Number | Output Resolution | Number | Output Resolution |
| --- | --- | --- | --- |
| 1 | 4096x2160p 60Hz | 8 | 1920x1080p 60Hz |
| 2 | 4096x2160p 50Hz | 9 | 1920x1080p 50Hz |
| 3 | 3840x2160p 60Hz | 10 | 1360x768p 60Hz |
| 4 | 3840x2160p 50Hz | 11 | 1280x800p 60Hz |
| 5 | 3840x2160p 30Hz | 12 | 1280x720p 60Hz |
| 6 | 3840x2160p 25Hz | 13 | 1280x720p 50Hz |
| 7 | 1920x1200p60Hz RB | 14 | 1024x768 60Hz |

User can select different output resolution by RS232 command

- Specification

| Band Width | 594MHz (18Gbps), HDMI 2.0, HDCP2,2 |
| --- | --- |
| Audio Format | LPCM, AC3, DD+, DTS, DTS-HD Up to 7.1 channel |
| Input ports | 2x HDMI，1x USB-C |
| Output ports | 1x HDMI 1x HDBaseT 1x 3.5mm LR audio 1x 5V/2A USB-A Charging port |
| Power Supply | 25W max |
| Operating Temperature | 0 to +40°C (+32 to +104 °F) |
| Operating Humidity | 10 to 70 % RH (non-condensing) |
| ESD | Air: ± 8KV, Contact: ± 4KV |

- Package Contents

| Item | Quantity |
| --- | --- |
| Hub Unit | 1 |
| AC Power Cord | 1 |
| User Manual | 1 |

- Mounting dimensions

- RS232 command

Note: All the commands begin with SET or GET, end with Carriage Return (CR).

Represents Carriage Return (CR). All return messages are end with CR

System command

| Command | Details |
| --- | --- |
| GET HELP | Get the Commands list |
| SET RESET | Recover to default setting |
| GET VERSION | Get firmware version Return: VERSION w (w is version number) |

Switching command

| Commands | Details |
| --- | --- |
| SET AUTO SWITCH w | w is ON or OFF, default OFF Return: AUTO SWITCH w |
| GET AUTO SWITCH | Return: AUTO SWITCH w |
| SET IN SOURCE w | w is one of the following: HDMI1, HDMI2, USB-C Return: IN SOURCE w |
| GET IN SOURCE | Get current input channel selection information Return: IN SOURCE w |
| GET IN RESOLUTION | Get current input resolution Return: IN RESOLUTION w (w is input resolution) |
| SET FOLLOW SWITCH w | w is ON or OFF, default ON Return: FOLLOW SWITCH w |
| GET FOLLOW SWITCH | Return: FOLLOW SWITCH w |

Output command

| Commands | Details |
| --- | --- |
| SET OUT RESOLUTION w | w is one of the following, default: 3840x2160p60 4096x2160p60, 4096x2160p50, 3840x2160p60, 3840x2160p50, 3840x2160p30, 3840x2160p25, 1920x1200p60RB, 1920x1080p60, 1920x1080p50, 1360x768p60, 1280x800p60, 1280x720p60, 1280x720p50, 1024x768p60 Return: OUT RESOLUTION w |
| GET OUT RESOLUTION | Get current output resolution setting Return: OUT RESOLUTION w |
| SET OUT HDCP w | w is one of following, default FORCE-1.4 FORCE-1.4,FORCE-2.2,FORCE-OFF Return: OUT HDCP w |
| GET OUT HDCP | Return: OUT HDCP w |
| SET OUT VKA w | w is BLUESCREEN or BLACKSCREEN. Default BLACKSCREEN. It is for no signal display Return: OUT VKA w |
| GET OUT VKA | Return: OUT VKA w |
| SET BLANK-OPT w | w is VKA or OSD, default VKA When select VKA, if user press front button BLANK or set RS232 command SET SHOWME OFF, then the screen will display VKA mode BLACKSCREEN or BLUESCREEN When select OSD, then after the same operation the screen will show OSD |
| GET BLANK-OPT | Return: BLANK-OPT w |
| SET SHOWME w | w is ON or OFF, default ON This command mute or unmute the video and audio output The function is the same as front BLANK button |
| GET SHOWME | Return: SHOWME w |
| SET OUT ITC w | w is ON or OFF, default OFF Return: OUT ITC w |
| GET OUT ITC | Return: OUT ITC w |

EDID command

The following commands are used to set EDID mode for the inputs

| Commands | Details |
| --- | --- |
| SET IN EDIDMODE w | w is one of the following: 4K60-2.0, 4K60-5.1, 4K60-7.1, 4K30-2.0, 4K30-5.1, 4K30-7.1, 1080p60-2.0,1080p60-5.1, 1080p60-7.1,1920x1200, 1680x1050, 1600x1200, 1440x900, 1360x768, 1280x1024, 1024x768, 720p, AUTO Default: 4K60-2.0 Return: IN EDIDMODE w |
| GET IN EDIDMODE | Return: IN EDIDMODE w |