QVM Series Matrix Switcher User Manual

1 card with 4 channels

Seamless Switching

Multiview

Video Wall Control

Simple Peripheral Controlling

Warning

- Do not expose this device to Rain, Moisture, and Dripping

- Only use accessories specified by the manufacture

- Unplug this device during Lightning Storms

- The manual is for reference only, maybe updated without further notice

Content

1. System description	2

1.1. Introduction	2

1.2. Features	2

1.3. Specification-main unit	3

1.4. Specification-input card and output card	3

1.5. Output resolution and HEX index	4

2. Panel introduction	5

2.1  Front view	5

2.2  Rear view	6

3. Package	7

4. PC Tool user guide	7

4.1  Connect tab	7

4.2  Matrix switch tab	8

4.3  Signal config tab	9

4.4  TV wall tab	9

4.5  Multiview tab	10

4.6  System tab	10

4.7  Peripheral tab	11

5. How to achieve multiview function of splicing wall	11

6. How to achieve point to point display with LED panel	11

- System description

  - Introduction

- QVM series modular matrix switcher is a high-performance ultra high definition video signal switching device that supports 4K@60Hz-444 processing of input, output, switching, multiview and wall splicing

- This product adopts a dual power redundancy design and a plug-in single card 4-way structure, which can support up to 8-80 signal inputs and 8-80 signal outputs

  - Features

- Support seamless switching between different input resolutions

- Comprehensive splicing display function, four screen window multiview function and zoom function

- Support synchronization for all output ports, there are two options: general mode and sync delay mode

- Supports 90°, and 180° rotation base on different output cards

- Support auto output resolution and user define output resolution, suitable for LED panel display

- RGB video processing, perfectly restoring true colors

- Input cards：HDMI/HDBaseT/Fiber card，up to 3840x2160/4096x2160@60Hz input resolution

- SDI card，up to 1080p60 input resolution

- Output cards：HDMI/HDBaseT/Fiber card，up to 3840x2160/4096x2160@60Hz output resolution

- Support audio insert on input card and audio extracting on output card

- Provide multiple control ports: front panel buttons, remote control, RS232, network and web

- Dual power redundancy design

  - Specification-main unit

| Chassis | 8x8, 2U | 16x16, 3U | 32x32, 5U | 80x80, 11U |
| --- | --- | --- | --- | --- |
| Front view |  |  |  |  |
| Rear view |  |  |  |  |
| Size（W*D*H） | 483x365x89mm | 483x365x134mm | 483x365x223mm | 483x365x512mm |
| Gross weight | 9Kg | 13Kg | 23Kg | 60Kg |
| Power module | 100W*2 or 200W*2 Redundancy | 200W*2 or 350W*2 Redundancy | 350W*2 or 500W*2 Redundancy | 350W*4 or 500W*4 Redundancy |

  - Specification-input card and output card

| Input card | Interface | Signal | Resolution |
| --- | --- | --- | --- |
|  | HDMI | HDMI,DVI | Up to 3840x2160@60 |
|  | RJ45 | HDbaseT | Up to 3840x2160@60 Support POC |
|  | BNC | SDI | 480i,576i,1080i50/60,720p50/60, 1080p24/25/29/30/50/60 |
|  | DVI | HDMI,DVI | Up to 1920x1200@60 If need external LR audio input，please use DVI to DVI+3.5mm adapter |
|  | VGA | VGA, YPbPr, CVBS | VGA input card，support following three signals VGA: From 640x480@60 to 1920x1200@60 YPbPr: Use VGA to YPbPr adapter 480i,576i,480p/576p，1080i50/60,720p50/60, 1080p50/60 CVBS: Use VGA to YPbPr adapter PAL,NTSC |
|  | Fiber | LC | Sing mode and single cable 480p to 3840x2160p60 Transmission distance 10 kilometers |
| Output card | Interface | Signal | Resolution |
|  | HDMI | HDMI,DVI | From 480p to 3840x2160@60 Different daughter cards maybe have different resolution capability |
|  | RJ45 | HDBaseT | From 480p to 3840x2160@60 Support POC Different daughter cards maybe have different resolution capability |
|  | BNC | SDI | 1080i50/60,720p50/60,1080p50/60 |
|  | DVI | HDMI,DVI | From 1024x768 to 1920x1200@60 If need stand alone LR audio output，please use DVI to DVI+3.5mm adapter |
|  | VGA | VGA, YPbPr, CVBS | VGA output card，support following three signals VGA: From 1024x768@60 to 1920x1200@60 YPbPr: Use VGA to YPbPr adapter 1080i50/60,720p50/60, 1080p50/60 CVBS: Use VGA to YPbPr adapter PAL,NTSC |
|  | Fiber | LC | Sing mode and single cable 480p to 3840x2160p60 Transmission distance 10 kilometers |

  - Output resolution and HEX index

| Output card type |  | 4K category | 2Kcategory | DVI | 3G SDI | VGA card |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI,HDBaseT, | HDMI,HDBaseT |  |  | Signal type |  |  |
|  |  | Fiber |  |  |  | VGA | YPbPr | CVBS |
| Resolution And Hex Index | 00 | 4096x2160p60 | 1920x1200p60 | 1920x1200p60 | 1920x1080p60 | 1920x1200p60 | 1920x1080p60 | PAL |
|  | 01 | 4096x2160p50 | 1920x1080p60 | 1920x1080p60 | 1920x1080i60 | 1920x1080p60 | 1920x1080i60 | NTSC |
|  | 02 | 3840x2160p60 | 1920x1080p50 | 1920x1080i60 | 1920x1080p50 | 1680x1050p60 | 1920x1080p50 |  |
|  | 03 | 3840x2160p50 | 1920x1080p30 | 1920x1080p50 | 1920x1080i50 | 1600x1200p60 | 1920x1080i50 |  |
|  | 04 | 3840x2160p30 | 1680x1050p60 | 1920x1080i50 | 1920x1080p30 | 1360x768p60 | 1280x720p60 |  |
|  | 05 | 3840x2160p25 | 1600x1200p60 | 1920x1080p30 | 1280x720p60 | 1280x1024p60 | 1280x720p50 |  |
|  | 06 | 3440x144p60 | 1360x768p60 | 1680x1050p60 | 1280x720p50 | 1280x768p60 |  |  |
|  | 07 | 2560x1600p60 | 1280x1024p60 | 1600x1200p60 |  | 1280x720p60 |  |  |
|  | 08 | 2560x1440p60 | 1280x768p60 | 1360x768p60 |  | 1024x768p60 |  |  |
|  | 09 | 1920x1200p60 | 1280x720p60 | 1280x1024p60 |  |  |  |  |
|  | 0A | 1920x1080p60 | 1280x720p50 | 1280x768p60 |  |  |  |  |
|  | 0B | 1920x1080p50 | 1024x768p60 | 1280x720p60 |  |  |  |  |
|  | 0C | 1920x1080p30 | USER | 1280x720p50 |  |  |  |  |
|  | 0D | 1680x1050p60 | AUTO | 1024x768p60 |  |  |  |  |
|  | 0E | 1600x1200p60 |  |  |  |  |  |  |
|  | 0F | 1360x768p60 |  |  |  |  |  |  |
|  | 10 | 1280x1024p60 |  |  |  |  |  |  |
|  | 11 | 1280x768p60 |  |  |  |  |  |  |
|  | 12 | 1280x720p60 |  |  |  |  |  |  |
|  | 13 | 1280x720p50 |  |  |  |  |  |  |
|  | 14 | 1024x768p60 |  |  |  |  |  |  |
|  | 15 | USER |  |  |  |  |  |  |
|  | 16 | AUTO |  |  |  |  |  |  |

- Panel introduction

2.1  Front view

- Installation screw: used to fix the control panel to the rack

- LCM display screen: displays the routing status of each channel in the matrix.

- Lock button : When activated, the inside light will be on, then all the buttons on the front panel will be locked and will not work. Press the button again, the light will turn off and unlock

- TAKE/ENTER button：Execution action

- OUT SELECT：Output 0, 1, 2...9, 10, 11... ,16

- IN SELECT：Input 0, 1, 2...9, 10, 11... ,16

Press OUTPUT m +INPUT n +TAKE,  switch input n to output m

- ALL button: Press ALL + INPUT n +TAKE, switch input n to all output ports

- SAVE button : Press SAVE+ OUTPUT n +TAKE, save current routing and screen layout to scene n

- RECALL button: Press RECALL+ OUTPUT n +TAKE, load scene for current displaying

- IR: IR receiver, can be disabled by RS232 command

- Directly press TAKE button, front LCD panel will loop display IR ON/OFF,RS232 Baud Rate and IP parameters

- 2.2  Rear view

| Index | Function | Description | Details |
| --- | --- | --- | --- |
| 1 | Main Control Use PC to control this matrix switcher | TCP/IP or Web Control | IP address:192.168.0.247 Submask:255.255.255.0 Gateway:192.168.0.1 Baud Rate:9600 Web login account : admin Web login password : admin |
| 2 |  | RS232 Control | Baud Rate:9600 |
| 3 |  | PS232-Phoenix control | T: Matrix Switcher -> PC G: Ground R: Matrix Switcher <- PC |
| 4 | Peripheral Control Control other peripherals through this matrix switcher | RS232-1 Peripheral Control | T: Matrix Switcher -> Peripheral G: Ground R: Matrix Switcher <- Peripheral |
| 5 |  | RS232-2 Peripheral Control | T: Matrix Switcher -> Peripheral G: Ground R: Matrix Switcher <- Peripheral |
| 6 |  | RS485 Peripheral Control | Connect to peripheral's RS485 port G: Ground |
| 7 |  | GPIO-Input | 4 inputs level detect,high or low G: Ground VIL : 0～2.0V VIH : 3.0～5.0V |
| 8 |  | GPIO-Output | 4 outputs,high or low G: Ground VOL : 0～0.5V VOH : 4.5～5.0V |
| 9 |  | Relay Control | 4 relays control, on or off G: Ground Rating: 48V/1A |

- Package

| Content | Quantity |
| --- | --- |
| Matrix switcher unit | 1 |
| AC cord | 2 |
| USB to RS232 cable | 1 |
| 1-meter net cable | 1 |

- PC Tool user guide

This PC tool is an installation free control software. It is divided into seven tabs according to different functions: Connect, Matrix, Signal, TV wall, System, Multiview and Peripheral.               The initial login password is: 111111

- 4.1  Connect tab

Please note：

- Use serial cable (straight line) or Ethernet cable

- When using a serial port connection, the network port connection must be disconnected, and vice versa

- When connecting through the network port, you must first search for and select the device before connecting

- 4.2  Matrix switch tab

- 4.3  Signal config tab

4.4  TV wall tab

This tab set the splicing wall parameters: layout, border, input/output settings, etc

- 4.5  Multiview tab

- 4.6  System tab

This tab sets network parameters, resets, reads software versions, etc

- 4.7  Peripheral tab

- How to achieve multiview function of splicing wall

Step 1, set a certain output card ( suggest to use the last output card ) to work on the multiview mode. We call one of the output ports on this card as output port M

Step 2, use one of the input ports of a certain input card ( suggest to use the last input port of the last input card ) as the multiview source input port, called input port N

Step 3, connect an external HDMI short cable between output port M and input port N

Step 4, set the parameters of the splicing wall and select the input source of the splicing wall as input N

- How to achieve point to point display with LED panel

Step 1, connect input cable between source and one input port of the matrix switcher.

Step 2, connect output cable between LED panel and one output port of the matrix switcher.

Step 3, read the EDID of LED panel with PC Tool and download this EDID to the input port of the matrix switcher.

Step 4, set the output resolution of this output port as AUTO, or USER. When set USER, need configure the user define resolution first, and make the user defined resolution to match the physical resolution of the LED panel perfectly